AMERICAN INTERNATIONAL UNIVERSITY–BANGLADESH (AIUB)

FACULTY OF SCIENCE & TECHNOLOGY

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

MICROPROCESSOR AND EMBEDDED SYSTEMS

Spring 2024-2025

Section: Q, Group: 08

LAB REPORT ON

Communication between two Arduino Boards using SPI

Supervised By

Protik Parvez Sheikh

Submitted By

MD. TANJIL TASHRIK ZIM

Group Member

| No. | Name | ID |
| --- | --- | --- |
| 1. | MD. TANJIL TASHRIK ZIM | 22-48021-2 |
| 2. | MD. AL-IMRAN SAYEM | 22-48023-2 |
| 3. | MD. ABRAR RAFID SHARIAR | 22-48055-2 |
| 4. | HUMAIRA ZANNAT | 22-47789-2 |
| 5. | SANJUKTA DAS | 22-47801-2 |

Date of Submission: May13, 2025

Title:  Communication between two Arduino Boards using SPI.

Objective: The objective of the "Communication between two Arduino Boards using SPI" experiment is to demonstrate the implementation of Serial Peripheral Interface (SPI) for inter-board communication in embedded systems. By connecting two Arduino boards via SPI, the experiment aims to establish a reliable and efficient data exchange mechanism. This involves configuring one Arduino as the master device and the other as the slave, enabling synchronous serial communication between them. Through this experiment, participants gain practical experience in utilizing SPI protocols for real-time data transfer, enabling applications such as sensor networks, control systems, and collaborative multi-device projects in the field of electronics and robotics.

Apparatus:

- Arduino UNO (2)

- LED (2)

- Push Button (2)

- Resistors 10 k, 2.2 k (2 + 2)

- Breadboard

- Connecting Wires

Experimental Setup:

Fig-1: Two Arduino board’s pin connections for SPI communications (schematic diagram)

Fig-2: Experimental setup

Code:

Code for Master Device

#include<SPI.h> //Library for SPI #define LED 7 #define ipbutton 2 int buttonvalue; int x; void setup (void){ Serial.begin(115200); //Starts Serial Communication at Baud Rate 115200 pinMode(ipbutton,INPUT); //Sets pin 2 as input pinMode(LED,OUTPUT); //Sets pin 7 as Output SPI.begin(); //Begins the SPI communication SPI.setClockDivider(SPI_CLOCK_DIV8); //Sets clock for SPI communication at // 8 (16/8 = 2 MHz) digitalWrite(SS,HIGH); //Setting SS to HIGH do disconnect master from slave } void loop(void){ byte Mastersend, Mastereceive; buttonvalue = digitalRead(ipbutton); //Reads the status of the pin 2 if(buttonvalue == HIGH) //Setting x for the slave based on input at pin 2 { x = 1; } else { x = 0; } digitalWrite(SS, LOW); //Starts communication with Slave from the Master Mastersend = x; Mastereceive = SPI.transfer(Mastersend); //Sends the Mastersend value to //the slave and also receives value from the slave if(Mastereceive == 1) //To set the LED based on value received from slaveu { digitalWrite(LED,HIGH); //Sets pin 7 HIGH Serial.println("Master LED is ON"); } else { digitalWrite(LED,LOW); //Sets pin 7 LOW Serial.println("Master LED is OFF"); } delay(1000); }

Code for Slave Device

#include<SPI.h> #define LEDpin 7 #define buttonpin 2 volatile boolean received; volatile byte Slavereceived, Slavesend; int buttonvalue; int x; void setup(){ Serial.begin(115200); pinMode(buttonpin,INPUT); // Setting pin 2 as INPUT pinMode(LEDpin,OUTPUT); // Setting pin 7 as OUTPUT pinMode(MISO,OUTPUT); //Sets MISO as OUTPUT to send data to Master In SPCR ¦= _BV(SPE); //Turn on SPI in Slave Mode received = false; SPI.attachInterrupt(); //Interrupt ON is set for SPI communication } ISR(SPI_STC_vect) //Interrupt routine function { Slavereceived = SPDR; // Value received from Master stored in Slavereceived received = true; //Sets received as True } void loop() { if(received) //To set LED ON/OFF based on the value received from Master { if (Slavereceived == 1) { digitalWrite(LEDpin, HIGH); //Sets pin 7 as HIGH to turn on LED Serial.println("Slave LED is ON"); } else { digitalWrite(LEDpin,LOW); //Sets pin 7 as LOW to turn off LED Serial.println("Slave LED is OFF"); } buttonvalue = digitalRead(buttonpin); //Reads the status of the pin 2 if (buttonvalue == HIGH) { x = 1; } else { x = 0; } Slavesend = x; SPDR = Slavesend; //To set the value of x to send to Master //Sends the x value to the Master via SPDR delay(1000); } }

Experimental  Results:

Fig-3: Connection between master and slave device

Fig-4: Clicked master device button and light                     Fig-5: Clicked salve device button and light on for

On for slave device                                                               Master device

Simulation Setup:

Fig-6: Simulation of SPI communication

Discussion and Conclusion:

Implementing communication between Arduino boards using SPI offers several advantages. SPI is a synchronous serial communication protocol that enables high-speed data transfer between devices, making it suitable for real-time applications. By designating one Arduino as the master and the other as the slave, precise control over data transmission is achieved, facilitating reliable communication. Additionally, SPI requires minimal hardware overhead, utilizing only a few pins for communication, which conserves resources and simplifies circuitry. This experiment enhances understanding of SPI configuration, including settings such as clock polarity, phase, and data order, which are crucial for successful communication.

However, there are limitations and considerations to acknowledge. SPI communication is typically point-to point, restricting the number of devices that can be connected directly. This can pose challenges in scenarios requiring communication among multiple nodes, necessitating additional hardware or alternative communication protocols. Moreover, while SPI offers high-speed communication, longer distances between devices may introduce signal degradation, requiring signal conditioning or the use of additional components.

In conclusion, the experiment demonstrates the effectiveness of SPI for inter-board communication in Arduino-based systems. Participants gain valuable insights into configuring SPI communication and leveraging its benefits for efficient data exchange. By understanding the nuances of SPI configuration and addressing its limitations, developers can effectively integrate SPI into their projects to enable seamless communication between Arduino boards. Overall, the experiment underscores the importance of selecting appropriate communication protocols and mastering their implementation for building robust and scalable embedded systems

Reference(s):

- https://www.arduino.cc/.

- ATMega328 manual

- https://www.avrfreaks.net

- http://maxembedded.com